| NO | Item/ Specifications | Unit | QTY per kit | Piece Unit Price in USD | Transportation & Delivery cost for each piece including loading& unloading | Total Price in USD ((Qty/Kit X Piece Unit Price X Transportation & Delivery cost for each piece including loading& unloading)) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Wall |  |  |  |  |  |
| 1a | Post - Wooden Pole: 75mm x 75mm x3000mm length, Margin of tolerance: section size: 75mm x75mm (± 5mm) length: 3000 mm (± 30 mm) Colour: white or brown. Quality: hard quality, heartwood Treatment: the wood must be certificated that was treated all parts of each pole against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of crack as much as possible. | No. | 8.00 |  |  |  |
| 1b | Girt Beam - Wooden Plate: 100mm width x 22mm depth x 4000mm length, Margin of tolerance: Width: 100 mm (±3 mm) Thickness: 22 mm (±2 mm) Length: 4000mm (± 30mm) Colour: white or brown. Quality: hard quality Treatment: the wood must be certificated that was treated all parts of each plate against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of cracks as much as possible | No. | 7.00 |  |  |  |
| 1c | Girt Beam - Wooden Plate: 100mm width x 22mm thickness x 3000mm length. Margin tolerance: Width: 100 mm (±3 mm) Thickness: 22 mm (±2 mm) Length: 3000mm (± 30mm) Colour: white or brown. Quality: hard quality Treatment: the wood must be certificated that was treated all parts of each plate against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of cracks as much as possible | No. | 6.00 |  |  |  |
| 1d | Thermal Insulation Foam: Roll 20m length x1.0m width x15mm thickness. Margin of tolerance: Width: 1000 mm (±10 mm) Thickness: 15 mm (±1 mm) Length: 20m (± 5cm) Colour: Plumbic or Silver. Quality: good quality | Roll | 1.50 |  |  |  |
| 1e | Plastic Sheet(4no. Walls): Dimensions: 4x6 yard (3.5x5.35m) Weight: approx. (3.5 - 4.5) kg Manufacturing: made of woven high-density black polyethylene fibers, with reinforced rims by heat sealing on all sides. Quality: good quality | No. | 4.00 |  |  |  |
| 1f | Nails: Steel galvanized round wire common nail (10D). Length: 3 Inch (75mm), Diameter: 5 mm, Box weight:1.0kg. Quality: good quality | Box | 1.00 |  |  |  |
| 1g | Door Frame - Wooden Plate: 100mm width x 22mm depth x 4000mm length, Margin of tolerance: Width: 100 mm (±3 mm) Thickness: 22 mm (±2 mm) Length: 4000mm (± 30mm) Colour: white or brown. Quality: hard quality Treatment: the wood must be certificated that was treated all parts of each plate against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of cracks as much as possible. Note: (plastic sheet shutter, measured under plastic sheet for wall) | No. | 1.00 |  |  |  |
| 1h | Door Frame - Wooden Plate: 100mm width x 22mm depth x 3000mm length, Margin of tolerance: Width: 100 mm (±3 mm) Thickness: 22 mm (±2 mm) Length: 3000mm (± 30mm) Colour: white or brown. Quality: hard quality Treatment: the wood must be certificated that was treated all parts of each plate against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of cracks as much as possible. Note: (plastic sheet shutter, measured under plastic sheet for wall) | No. | 1.00 |  |  |  |
| 1i | Door Hinges: Dimensions: 100mm length, 50mm width with at least 3 holes for bolts. Quality: good quality staineless steel hinges, and complete with fixing screws. | No. | 3.00 |  |  |  |
| 1j | Door Latch Barrel Bolt: Dimension: 100mm long (internal and external locks) Quality: good quality stainless steel straight barrel latch bolt, and complete with fixing screws . | No. | 2.00 |  |  |  |
| 2 | Roof |  |  |  |  |  |
| 2a | Rafter - Wooden Pole: 50mm x 50mm x4000mm length, Margin of tolerance: Section size: 50mm x50mm (± 3mm) Length: 3000 mm (± 30 mm) Colour: white or brown. Quality: hard quality, heartwood Treatment: the wood must be certificated that was treated all parts of each pole against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of crack as much as possible. | No. | 4.00 |  |  |  |
| 2b | Purline - Wooden Plate: 100mm width x 22mm depth x 4000mm length, Margin of tolerance: Width: 100 mm (±3 mm) Thickness: 22 mm (±2 mm) Length: 4000mm (± 30mm) Colour: white or brown. Quality: hard quality Treatment: the wood must be certificated that was treated all parts of each plate against the termite and insects; chemicals used for treatment must pose no risk to human health. Moisture Level: dry as much as possible with moisture level (if any) must be between 9% to 15%. Shake, Cracks, free of cracks as much as possible | No. | 4.00 |  |  |  |
| 2c | Plastic Sheet(1No.Roof): Dimensions: 4x6 yard (3.5x5.35m) Weight: approx. (3.5 - 4.5) kg Manufacturing: made of woven high-density black polyethylene fibers, with reinforced rims by heat sealing on all sides. Quality: good quality | No. | 1.00 |  |  |  |
| 2d | Thermal Insulation Foam: Roll 20m length x1.0m width x15mm thickness. Margin of tolerance: Width: 1000 mm (±10 mm) Thickness: 15 mm (±1 mm) Length: 20m (± 5cm) Colour: Plumbic or Silver. Quality: good quality | Roll | 1.00 |  |  |  |
| 2e | Nails - Umberalla Type: Steel galvanized round wire common nail (6D). Length:2 Inch (50mm), Diameter: 3 mm, Box weight:1.0kg. Quality: good quality | Box | 1.00 |  |  |  |
| 2f | Sisal Rope Length: 30m length Margin of tolerance: Length: 30m (± 0.5 m). Thickness: 5 mm (± 0.1 mm) Type: Natural Fiber | Roll | 1.00 |  |  |  |
| 2G | Nylon Rope Length: 30m length Margin of tolerance: Length: 30m (± 0.5 m). Thickness: 7 mm (± 0.3 mm) Type: Nylon | Roll | 1.00 |  |  |  |
| 2H | Nylone Zip Cable Tie: To ( attached thermal insulation to wooden plates/poles). Margin of tolerance: Length: 20-inch (600 mm) (± 10) Width: 4mm, (± 0.5mm) Box: with 100 pcs Heavy Duty Nylon Cable Tie Wire Zip Ties Self Locking Tie Wraps Cable | Box | 1.00 |  |  |  |
| 3 | Tools |  |  |  |  |  |
| 3a | Saw: Size: 610 mm blade (hard point teeth), Handle: plastic | pcs | 1.00 |  |  |  |
| 3b | Hammer: Size: 4 inch ('l0cm) handle with finger grooves. Head: steel head, magnetic, Type: claw with rubber handle. | pcs | 1.00 |  |  |  |
| 3c | Pickaxe: Size: 0.55 meters (incl. digger head). Handle type: flat / pick edge. | pcs | 1.00 |  |  |  |
| 3d | Utility knife: Size: 100x18x0.5 mm. Type: steel series. Handle: plastic. | Pcs | 1.00 |  |  |  |
| 3e | Measure Tape Meter: Type: metal measure tape Length: 5m Brand: Good Quality | Pcs | 1.00 |  |  |  |
| 3F | Shovel: Type: Wooden stick with metal handle Head width: 300mm (± 30 mm) Head Length: 330mm (± 30 mm) Length: 1000 mm (± 100 mm) Brand: Good Quality | Pcs | 1.00 |  |  |  |
| Total price in USD including proper packaging (kit),printing logos(Stickers), transportation, delivery and loading and unloading at first and end point and to/from trucks and at final destination assigned by DRC in any districts in Shabwa or Al-Dhale’a governorates |  |  |  |  |  |  |

Note: All wooden components must be treated with a certified anti-white termite solution to ensure long-term durability and protection against termite infestations. DRC would verify on site upon delivery.